HJJDS6031

Parameter description:	2

Appearance	3

Introduction to Interface Display	5

Key	6

Operation introduction	7

Press down CH:	7

Press down PARM:	8

Press down TRIG:	8

First press down HORI:	9

Second press down HORI:	10

First press down MENU:	10

Second press down MENU:	11

Third press down MENU:	12

switch signal generator	12

Program upgrade (U disk mode)	13

# Parameter description:

Channel:  					1

Bandwidth:					50MHz

real-time sampling rate     		200MSa/s

The storage depth         		4Kpts

Input coupling            		AC、DC

Input impedance          		1MΩ 25pF

The maximum input voltage 		40V (probe X1); 400V (probe X10) can be measured 220V

voltage; (probe X100) 2000V voltage can be measured

Probe attenuation             	1X、10X

Set the probe attenuation factor	1X、10X、100X

Sampling Method				Real-time sampling(10ns-50ms)/Scan sampling(100ms-5s)

Vertical Sensitivity 				10mV-5V (Probe 1X) 100mV-50V (probe 10X) (1,2.5,5 step)

Vertical accuracy				+/-3%

Vertical resolution				8bit

Horizontal scan range			10nS/div-5S/div(1,2.5,5 step)

Trigger Mode					Auto, Normal and Single

Trigger Type					Rising edge trigger, falling edge trigger

Automatic detection 			Support (50Hz-30MHz)

Cursor measurements			Support time and voltage cursors

Signal generator

waveform					Square wave, Sine wave, Linear voltage

frequency					1Hz - 40KHz

Amplitude					5V(MAX-+2.5V),

offset						MAX--+2.5V

Square wave duty cycle			0% - 100%(Minimum resolution1%)

Screen 						3.2-inch, 16-bit true color, TFT, 320 * 240

Battery						2800 + mA lithium battery (single cell about four Six of

continuous work)

Size 						195 * 99* 39(mm)

# Appearance

# Introduction to Interface Display

# Key

Main buttons as follows

CH						Access Channel Control

PARM					Display parameters

Power					Turn on and off

AUTO					Automatic retrieval of waveforms

TRIG						Trigger control

HORI					Time-based correlation control

STOP/RUN				Stop start waveform display

MENU					Control menu

OK						Under the corresponding menu Function key

↑						Under the corresponding menu Function key

↓						Under the corresponding menu Function key

←						Under the corresponding menu Function key

→						Under the corresponding menu Function key

F1						Under the corresponding menu Function key

F2						Under the corresponding menu Function key

F3						Under the corresponding menu Function key

# Operation introduction

Press down CH:

Function key description

↑						Voltage shift control

↓						Voltage shift control

←						Waveform Horizontal Position Control

→						Waveform Horizontal Position Control

F1						Open Closed Channel Display

F2						probe x1,x10,x100

F3						AC and DC

Press down PARM:

Function key description

↑						Voltage shift control

↓						Voltage shift control

←						Waveform Horizontal Position Control

→						Waveform Horizontal Position Control

F1						Open Closed Channel Display

F2						probe x1,x10,x100

F3						AC and DC

Press down TRIG:

Function key description

↑						Trigger position shift

↓						Trigger position shift

←						Trigger position shift

→						Trigger position shift

F1						Rising edge and falling edge trigger

F2						Choose whether to enable smart trigger (automatically find the 							trigger position)

F3						Select trigger type automatically normal single

First press down HORI:

Function key description

↑						Changing time base

↓						Changing time base

←						Change the position of the time base arrow

→						Change the position of the time base arrow

F1						Open and close measuring scale

F2						The object of scale measurement

F3						Type Voltage Time Measured by Scale

Second press down HORI:

Function key description

↑						Position of scale 2 of the table

↓						Position of scale 2 of the table

←						Position of scale 1 of the table

→						Position of scale 1 of the table

F1						invalid

F2						invalid

F3						invalid

First press down MENU:

Function key description

↑						invalid

↓						invalid

←						invalid

→						invalid

F1						Label of stored waveform A or B

F2						Source of stored waveform

F3						Whether to display stored waveforms

Second press down MENU:

Function key description

↑						invalid

↓						invalid

←						invalid

→						invalid

F1						Select whether the channel display is inverted

F2						Select language type

F3						sound switch

Third press down MENU:

Function key description

↑						invalid

↓						invalid

←						invalid

→						invalid

F1						Self-calibration (note that all signal connections are disconnected 						before calibration)

F2						Screenshot related options

F3						Press OK to reset to factory settings

OK Explain:

Used for screenshots when screenshots are turned on

Used to store waveforms when screenshots are turned off

switch signal generator

Long press the MENU button on the oscilloscope or signal generator interface to switch between the two functions

Signal generator interface and operation

Function key introduction

↑						Modify selected numbers

↓						Modify selected numbers

←						Select the number to be modified

→						Select the number to be modified

F1						Select frequency/offset

F2						select waveform

F3						Select Amplitude/Duty Cycle

Program upgrade (U disk mode)

After the shutdown, press the OK button and the power button at the same time to enter the U disk mode (the U disk will appear on the PC end after connecting the usb cable)

Copy the upgrade file update.bin into the U disk, and press the MENU button to upgrade.